Adding the MCS7720 drivers to the Platform

Introduction:

This document provides the information about how to add/include MCS7720 device drivers to a windows CE .NET 4.2 platform.

The following steps explains how to add/include the MCS7720 device drivers to Windows CE .NET 4.2 platform.

- Create and Build a CEPC platform

- After building the platform add the following entries in the platform.reg file

[HKEY_LOCAL_MACHINE\Drivers\USB\LoadClients\38672_30496\255_0_255\Default\Mos20UsbSerialClass]

"Dll"="USer20Ce.DLL"

[HKEY_LOCAL_MACHINE\Drivers\USB\ClientDrivers\Mos20UsbSerialClass]

"Dll"="USer20Ce.DLL"

(Note: Entries should be there above the following line in the platform.reg

; END HIVE BOOT SECTION

; ---------------------------- BEGIN PCI DEVICES ----------------------------)

(Please see the .\7720Others\Platform.reg for reference)

- Add the following entries in the platform.bib file above the following line

User20Ce.dll      $(_FLATRELEASEDIR)\User20Ce.dll             NK  SH

(Note: Entries should be there above the following line in the platform.bib

FILES

; Name            Path                                           Memory Type)

(Please see the .\7720Others\Platform.bib for reference)

- Copy the User20Ce.dll in to the following folder.

Drive:>\WINCE420\PUBLIC\CEPlatform\RelDir\CEPC_X86Release

- Then build the platform to effect the above changes.

This platform image contains the MCS7720 device driver and you can connect MCS7720 to the target machine and configure after downloading the platform OS image.